# Intermodal freight transport

### From Wikipedia, the free encyclopedia

Jump to: navigation, search

For passenger transport, see Intermodal passenger transport.

|  | Look up intermodal freight transport in Wiktionary, the free dictionary. |
| --- | --- |

|  | The examples and perspective in this article may not represent a worldwide view of the subject. Please improve this article and discuss the issue on the talk page. |
| --- | --- |

Intermodal ship-to-rail transfer of containerized cargos at APM Terminals in Portsmouth, VA.

Intermodal freight transport involves the transportation of freight in an intermodal container or vehicle, using multiple modes of transportation (rail, ship, and truck), without any handling of the freight itself when changing modes. The method reduces cargo handling, and so improves security, reduces damages and losses, and allows freight to be transported faster. Reduced costs versus over road trucking is the key benefit for intracontinental use.

## Contents [hide] - 1 Prehistory - 2 History - 2.1 Double-stacked container transport - 3 Equipment - 3.1 Containers - 3.2 Handling equipment - 4 Load Securing in Intermodal Containers - 5 Intermodal Vehicles - 5.1 Rail transport - 5.2 Container ships - 5.3 Trucking - 5.4 Barges - 6 Current developments - 6.1 2009 - 7 See also - 8 References - 9 Bibliography - 10 External links

## [edit] Prehistory

Intermodal transportation goes back to the 18th century and predates the railways. Some of the earliest containers were those used for shipping coal on the Bridgewater Canal in England in the 1780s. Coal containers (called 'loose boxes') were soon deployed on the early railways and used for road/rail transfers (road at the time meaning horse drawn vehicles).

Wooden coal containers used on railways go back to the 1830s on the Liverpool and Manchester Railway. In 1841 Isambard Kingdom Brunel introduced iron containers to move coal from the vale of Neath to Swansea Docks. By the outbreak of the First World War the Great Eastern Railway was using wooden containers to trans-ship passenger luggage between trains and sailings via the port of Harwich.

The early 1900s saw the first adoption of covered containers, primarily for the movement of furniture and intermodal freight between road and rail. A lack of standards limited the value of this service and this in turn drove standardisation. In the USA such containers, known as "lift vans', were in use from as early as 1911.

## [edit] History

An intermodal train carrying both shipping containers and highway semi-trailers in "piggyback" service, on flatcars, passes through the Cajon Pass in February, 1995.

In the United Kingdom containers were first standardised by the Railway Clearing House (RCH) in the 1920s, allowing both railway owned and privately owned vehicles to be carried on standard container flats. By modern standards these containers were small, being 1.5 or 3.0 meters long (5 or 10 ft), normally wooden and with a curved roof and insufficient strength for stacking. From 1928 the London, Midland and Scottish Railway offered 'door to door' intermodal road-rail services using these containers. This standard failed to become popular outside the United Kingdom.

Pallets made their first major appearance during World War II, when the United States military assembled freight on pallets, allowing fast transfer between warehouses, trucks, trains, ships, and aircraft. Because no freight handling was required, fewer personnel were required and loading times were decreased.

Truck trailers were first carried by railway before World War II, an arrangement often called "piggyback", by the small Class I railroad, the Chicago Great Western in 1936. The Canadian Pacific Railway was a pioneer in piggyback transport, becoming the first major North American railway to introduce the service in 1952. In the United Kingdom the big four railway companies offered services using standard RCH containers which could be craned on and off the back of trucks. Moving companies such as Pickfords offered private services in the same way.

In the 1950s container use started to revolutionize freight transportation. The United States Department of Defense produced specifications for standard containers for military use of 8-foot (2.4 m) by 8-foot (2.4 m) square cross section in units of 10-foot (3.0 m) long. The International Organization for Standardization (ISO) issued standards based upon the US Department of Defense standards between 1968 and 1970, ensuring interchangeability between different modes of transportation worldwide. These rectangular stackable containers became known as ISO containers.

One pioneering railway was the White Pass and Yukon Route, which acquired the world's first container ship, the Clifford J. Rogers, built in 1955, and introduced containers to its railway in 1956. In the United Kingdom the modernisation plan and in turn the Beeching Report strongly pushed containerisation. The British Railways freightliner service was launched carrying 8-foot (2.4 m) high pre-ISO containers. The older wooden containers and the pre-ISO containers were rapidly replaced by 10-foot (3.0 m) and 20-foot (6.1 m) ISO standard containers, and later by 40-foot (12 m) containers and larger.

In the USA, starting in the 1960s the use of containers increased steadily. Rail intermodal traffic tripled between 1980 and 2002, according to the Association of American Railroads (AAR), from 3.1 million trailers and containers to 9.3 million. Large investments were made in intermodal freight projects. An example was the USD $740,000,000 Port of Oakland intermodal rail facility begun in the late 1980s.[1][2]

### [edit] Double-stacked container transport

Intermodal ship-to-rail transfer of containerized cargos at the Port in Long Beach, California.

Since 1984, a mechanism for intermodal shipping known as double-stack rail transport has become increasingly common. Rising to the rate of nearly 70% of United States intermodal shipments, it transports more than one million containers per year. The double-stack rail cars design significantly reduces damage in transit and provides greater cargo security by cradling the lower containers so their doors cannot be opened. A succession of large, new domestic container sizes was introduced to increase shipping productivity. In Europe the more restricted loading gauge has limited the adoption of double-stack cars. However, in 2007 the Betuweroute was completed, a railway from Rotterdam to the German industrial heartland, which may accommodate double stacked containers in the future. Other countries, like New Zealand, have numerous low tunnels and bridges, which limits expansion for economic reasons.

Since electrification generally predated double stacking, the overhead wiring was too low to accommodate it. However, India is building some freight only corridors with the overhead wiring at 7.45 m above rail, which is high enough. [3]

## [edit] Equipment

### [edit] Containers

Main article: Intermodal freight shipping container

Containers at Kuantan Port

Containers being transported by rail in the UK, passing Northampton on February 18, 2008.

Containers, also known as intermodal containers or ISO containers because the dimensions have been defined by ISO, are the main type of equipment used in intermodal transport, particularly when one of the modes of transportation is by ship. Containers are 8-foot (2.4 m) wide by 8-foot (2.4 m) high. Since introduction, there have been moves to adopt other heights, such as 8-foot-6-inch (2.59 m), 9-foot-6-inch (2.90 m) and 10-foot-6-inch (3.20 m). The most common lengths are 20 feet (6.1 m) nominal or 19 feet (5.8 m) - 10+1⁄2 in (0.27 m) actual, 40 feet (12 m), 48 feet (15 m) and 53 feet (16 m), although other lengths exist. They are made of steel and can be stacked atop one another (a popular term for a two-high stack is "double stack").

ISO-code and dimension/load table on several newly washed containers

Onboard ships they are typically stacked up to seven units high. They can be carried by truck, rail, container ship, or aeroplane. When carried by rail, containers can be loaded on flatcars or in container well cars. In Europe, stricter railway height restrictions (smaller loading gauge and structure gauge) and overhead electrification prevent containers from being stacked two high, and containers are hauled one high either on standard flatcars or other railroad cars. Taller containers are often carried in well cars (not stacked) on older European railway routes where the loading gauge is particularly small.

Some variations on the standard container exist. Open-topped versions covered by a fabric curtain are used to transport larger loads. A container called a tanktainer, with a tank inside a standard container frame, carries liquids. Refrigerated containers are used for perishables. There is also the swap body, which is typically used for road and rail transport. Built too lightly to be stacked, they have folding legs under their frame so that they can be moved between trucks without using a crane.

Various non-standard container forms are commonly used. These include non-stackable open box containers, and several slightly non-standard geometries. European containers are often about two inches wider than the ISO standard although otherwise conformant, which can carry the euro-pallet standard pallet load. Specialised containers used in Europe include containerised coal carriers, and recently 'bin-liners' - containers designed for the efficient road/rail transportation of rubbish from cities to recycling and dump sites.

In countries where the loading gauge is sufficient, truck trailers are often used for freight that is transported primarily by road and rail. Typically, regular semi-trailers can be used, and do not need to be specially designed.

### [edit] Handling equipment

Straddle carriers in operation at the Port of Melbourne, Australia

Handling equipment can be designed with intermodality in mind, assisting with transferring containers between rail, road and sea. These can include:

- Transtainers for transferring containers from sea-going vessels onto either trucks or rail wagons. A transtainer is mounted on rails with a large boom spanning the distance between the ship's cargo hold and the quay, moving parallel to the ship's side.[4]

- Gantry cranes, also known as straddle carriers, are able to straddle rail and road vehicles, allowing for quick transfer of containers. A spreader beam moves in several directions allowing accurate positioning of the cargo.[4]

- Grappler lift, which is very similar to a straddle carrier

- Reach stackers are fitted with lifting arms as well as spreader beams and lift containers to swap bodies or stack containers on top of each other.[4]

[show] v • d • e Types of Cranes. Container crane · Crane vessel · Gantry crane · Level luffing crane · Sidelift · Straddle carrier · Tow truck.

## [edit] Load Securing in Intermodal Containers

| Application in container | Polyester Strapping and Dunnage Bag application | Polyester Lashing Application |
| --- | --- | --- |

According to the European Commission Transportation Department “it has been estimated that up to 25% of accidents involving trucks can be attributable to inadequate cargo securing”.[5] Cargo that is improperly secured can cause severe accidents and lead to the loss of cargo, the loss of lives, the loss of vehicles, ships and airplane; not to mention the environmental hazards it can cause. There are many different ways and materials available to stabilize and secure cargo in containers used in the various modes of transportation. Conventional Load Securing methods and materials such as steel banding and wood blocking & bracing have been around for decades and are still widely used. In the last few years the use of several, relatively new and unknown Load Securing methods have become available through innovation and technological advancement including polyester strapping and -lashing, synthetic webbings and Dunnage Bags, also known as air bags.

## [edit] Intermodal Vehicles

### [edit] Rail transport

A portion of a "double stack" container train operated by Union Pacific Railroad, the containers are owned by Pacer Stacktrain.

In North America, containers are often shipped by rail in container well cars. These cars resemble flatcars but the newer ones have a container-sized depression, or well, in the middle (between the bogies or "trucks") of the car. This depression allows for sufficient clearance to allow two containers to be loaded in the car in a "double stack" arrangement. The newer container cars also are specifically built as a small articulated "unit", most commonly in components of three or five, whereby two components are connected by a single bogie as opposed to two bogies, one on each car. (The photo above under "Equipment" shows an example of the new setup.) Double stacking is also used in parts of Australia. On some older railways, particularly in the United Kingdom, the use of well cars is necessary to carry single stacked large containers within the loading gauge.

It is also common in North America to transport semi-trailers on railway flatcars or spine cars, an arrangement called "piggyback" or TOFC (trailer on flatcar) to distinguish it from container on flatcar (COFC). Some flatcars are designed with collapsable trailer hitches so they can be used for trailer or container service.[6] Such designs allow trailers to be rolled on from one end, though lifting trailers on and off flatcars by specialized loaders is more common. TOFC terminals typically have large areas for storing trailers pending loading or pickup. Example.

A newer method of transporting trailers, the RoadRailer, has been developed by Wabash National. When the trailers are transported on rail, railway wheel assemblies are placed between the trailers, in effect turning the trailers into one large articulated railway car. This method is faster than loading trailers on flatcars and requires no extra railway cars, but they need to be specially designed (strengthened) to withstand the forces of being carried in a train of up to 150 trailers. One of the most well known operators of RoadRailers in the United States is Triple Crown Services, a subsidiary of Norfolk Southern Railway. Triple Crown operates a fleet of over 6500 Roadrailers and serves 14 terminals in the eastern half of the country.[7] A similar technology has been developed by RailRunner.

[show] v • d • e Rail transport freight equipment. Enclosed equipment; Autorack · Boxcar (US) · Coil car · Container · Covered hopper · Covered wagon · Refrigerator car (US) · Refrigerated van (EU) · Roadrailer · Stock car · Tank car. Open equipment; Flatcar · Gondola (US) · Hopper car · Open wagon (EU) · Schnabel car · Double-stack car. Non-revenue equipment; Caboose (US) · Brake van (EU) · Maintenance of way cars · Scale test car · Clearance car · Crane (railroad).

### [edit] Container ships

The 300-meter-long container ship CMA CGM Balzac

Container ships are used to transport containers by sea. These vessels are custom-built to hold containers. Some vessels can hold thousands of containers. Their capacity is often measured in TEU or FEU. These initials stand for "twenty-foot equivalent unit," and "forty-foot equivalent unit," respectively. For example, a vessel that can hold 1,000 40-foot containers or 2,000 20-foot containers can be said to have a capacity of 2,000 TEU. In the year 2005, the largest container ships in regular operation are registered to carry in excess of 8,000 TEU.

A key consideration in the size of container ships is that larger ships exceed the capacity of important sea routes such as the Panama and Suez canals. The largest size of container ship able to traverse the Panama canal is referred to as Panamax, which is presently around 5,000 TEU. A third set of locks is planned as part of the Panama Canal expansion project to accommodate container ships up to 12,000 TEU in future, comparable to the present Suezmax.

It costs about $8000[who?][when?] to ship a container from East Asia to North America[where?] when oil is at $100/barrel. As the price of oil increases, shipping costs increase.

Very large container ships also require specialized deep water terminals. The container fleet available, route constraints, and terminal capacity play a large role in shaping global container shipment logistics.[8][9]

[show] v • d • e Modern merchant vessels. Dry cargo; Barge · Bulk carrier · Car float · Coaster · Collier · Container ship · Hopper barge · Heavy lift ship · Lighter aboard ship · Reefer ship · RORO ship · Train ferry. Tankers; Oil tanker · Chemical tanker · LNG carrier · FPSO unit. Passenger; Cargo liner · Cruise ship · Cruiseferry · Ferry · Train ferry · Ocean liner. Support; Dive support · Fireboat · Supply ship · Tender · Tugboat. Other; Cable layer · Crane vessel · Drillship · Dredger · Fishing vessel · Icebreaker · Merchant submarine · Research vessel · Semi-submersible. Sizes, smallest to largest: Handysize · Handymax · Panamax · Aframax · Suezmax · Capesize · VLCC · ULCC Other sizes: Seawaymax · Malaccamax.

### [edit] Trucking

A truck transporting a container on Interstate 95 in South Florida.

Trucking is frequently used to connect the "linehaul" ocean and rail segments of a global intermodal freight movement. This specialized trucking that runs between ocean ports, rail terminals, and inland shipping docks, is often called drayage, and is typically provided by dedicated drayage companies or by the railroads.[10]

### [edit] Barges

Barges utilise ro-ro vessels to transport freight on large inland waterways such as the Rhine/Danube in Europe and the Mississippi River in the USA.[4]

## [edit] Current developments

### [edit] 2009

- Norfolk Southern to build $95 million intermodal terminal at Greencastle, Pennsylvania as part of its Crescent Corridor expansion project.[11]

- Work began in August 2009 to build $230 million intermodal terminal in North Baltimore, Ohio as part of the CSX National Gateway improvement project.[12][13]

## [edit] See also

| - The notion of co-modality introduced by the European Commission - Container Numbering - Containerization - Dunnage Bags - Double-stack car - Dry port - Inland port - Intermodal container - Load Securing | - Intermodal flatcars - Less-Than-Truckload (LTL) Shipping - Roadrailer - Rolling Highway - Shipping - Tanktainers - Transloading - Well car |
| --- | --- |

## [edit] References

- ^ Initial Study: Intermodal Interface Demonstration Project, Port of Oakland, Oakland, California, Earth Metrics and Korve Engineerning, December 20, 1989

- ^ Port of Oakland Official Site: Facts and Figures (2006)

- ^ Railway Gazette International August 2009, p. 17

- ^ a b c d Rushton, A., Oxley, J., Croucher, P. (2004) The Handbook of Logistics and Distraibution Kogan: London

- ^ "European Commission Transport". Road Safety: Best practice guidelines on cargo securing and abnormal transport. Updated April 9th, 2009. http://www.ec.europa.eu/transport/road_safety/vehicles/guidelines_cargo_securing__en.htm. Retrieved November 16th, 2009.

- ^ Atchison, Topeka & Santa Fe TOFC / COFC Cars Album

- ^ Triple Crown Services Company - RoadRailer Fleet

- ^ The Scottish Government - Container Transhipment and Demand for Container Terminal Capacity in Scotland

- ^ Halifax Port Authority Presentation

- ^ Forbes Logistics 2006

- ^ Norfolk Southern Corporation (2009). "Norfolk Southern Selects Greencastle Site for New Franklin County Regional Intermodal Facility, Supporting Crescent Corridor Initiative." Press release. 2009-08-12.

- ^ CSX Corporation (2009). "Ohio Governor Strickland, Congressman Latta, and CSX Chairman Ward Break Ground for Northwest Ohio." Press release. 2009-08-14.

- ^ Railway-Technology.com (2009). "Work Begins on Northwest Ohio Intermodal Terminal." 2009-08-19.

## [edit] Bibliography

- DeBoer, David J. (1992). Piggyback and Containers: A History of Rail Intermodal on America's Steel Highway. Golden West Books, San Marino, CA. ISBN 0-87095-108-4.

- European Intermodal Association (2005). Intermodal Transport in Europe. EIA, Brussels. ISBN 9090199136

- Sidney, Samuel (1846). Gauge Evidence: The History and Prospects of the Railway System. Edmonds, London, UK. No ISBN.

## [edit] External links

- IANA: The Intermodal Association of North America

- EIA: European Intermodal Association

- IARO: The International Air Rail Organisation

- Matt Hannes: The Intermodal Container Web Page

- Load Securing Supplier

- Railex The refrigerated Unit Train

[show] v • d • e Intermodal containers. Overview; Containerization · Intermodal freight transport · Twenty-foot equivalent unit · International Container Bureau · BIC code. Transport; Container ship · Well car. Handling; Container port · Container crane · Straddle carrier · Sidelifter · Rubber tyred gantry crane · Twistlock. Types; Refrigerated container · SECU. Other; Container Shipping Information Service · The Box · BBC Box · Sun Modular Datacenter · Shipping container architecture · Container (board game).

[show] v • d • e Transport of hydrocarbon fuels. Oil; Oil tanker · Oil storage · Tank car · Tank truck · List of oil pipelines. LNG and LPG; LNG carrier {Regasification · Liquefaction · Gas separation} · Natural gas storage · Black powder in gas pipelines · List of natural gas pipelines · List of LNG terminals. Coal/Misc.; Industrial railway · Intermodal freight transport · Syngas.

Retrieved from "http://en.wikipedia.org/wiki/Intermodal_freight_transport"

Categories: Commercial item transport and distribution | Intermodal containers