Warming Through the Main Turbines Prior to Standby Leaving Port

Once confirmation is received that the loading arms have been disconnected and are clear of the vessel, proceed as follows:

- Start raising main steam line pressure to 6.0Mpa

(utilizing the bypass valves on the main stop valves SS01F and SS02F, over a minimum of 20 minutes)

If main steam line is at operating pressure,

- Open fully the main stop valves SS01F and SS02F and

- Close the bypass valves.

- Close the main turbine warming through valves ST001F, ST012F, SS906F and SS907F and the main turbine extra nozzle valves

(unless the Chief Engineer indicates an MCR requirement)

*MCR refers to Maximum Continuous Rating of engine.

- Stop the gear casing dehumidifier.

- Stop and disengage the turning gear.

- Reset the main turbine emergency trip located at the turbine side emergency gauge board.

- Activate  CAUSE RESET pushbutton on the Manoeuvring Control Cabinet.

- Engage green Warming Through Overspeed protection pushbutton on the main turbine panel on the main console.

- Ensure Engineer is positioned at main turbine side

- Place the main turbine on ECR and Direct Control.

- Turn the Main turbine ahead and astern at a maximum of 8rpm using DIRECT CONTROL switch.

Once confirmation is received from main turbine side that all is in good order, continue to turn the main turbine ahead and astern.

- During this period, ensure the engine telegraph is tested from the bridge.

- When permission is received from the bridge/terminal that the gangway is removed, change over the main engine control to the ECR LEVER CONTROL position and switch on the AUTOSPIN.

- Confirm AUTOSPIN engages after 3 minutes and that the turbine turns ahead and astern sequentially.

- At Standby Engines, transfer the main turbine control mode from ECR to Wheelhouse.

Entry is made in the Engine Room Movement Book for the following items, indicating the event and the time they were completed:

  - Loading arms all disconnected and permission granted to commence warming through the main turbine.

  - Main turbine stop valves open.

  - Turning  gear stopped and disengaged.

  - Main turbine trip tested.

  - Main turbine warning through in ECR Direct Control.

  - Main turbine on ECR Lever Control and Autospin active.

  - Main Engine on Wheelhouse lever control.