## Marine Fuel Oil Classification

## [Ref: http://en.wikipedia.org/wiki/Fuel_oil ]

In the maritime field another type of classification is used for fuel oils:

- MGO (Marine gas oil) - roughly equivalent to No. 2 fuel oil, made from distillate only

- MDO (Marine diesel oil) - A blend of gas oil and heavy fuel oil

- IFO (Intermediate fuel oil) A blend of gas oil and heavy fuel oil, with less gas oil than marine diesel oil

- MFO (Medium fuel oil) - A blend of gas oil and heavy fuel oil, with less gas oil than intermediate fuel oil

- HFO (Heavy fuel oil) - Pure or nearly pure residual oil, roughly equivalent to No. 6 fuel oil

Marine diesel oil contains some heavy fuel oil, unlike regular diesels. Also, marine fuel oils sometimes contain waste products such as used motor oil.

### Standards and classification

Marine fuels were traditionally classified after its kinematic viscosity. This is a mostly valid criterion for the quality of the oil as long as the oil is made only from atmospheric distillation. Today, almost all marine fuels are based on fractions from other more advanced refinery processes and the viscosity itself says little about the quality as fuel. It has been developed two indexes CCAI and CII which describe the ignition quality of residual fuel oil, and especially CCAI are often calculated for marine fuels.

The density is also an important parameter for fuel oils since marine fuels are purified before use to remove water and dirt from the oil. Since the purifiers use centrifugal force, the oil must have a density which is sufficiently different from water. Older purifiers had a maximum of 991kg/m3; with modern purifiers it is also possible to purify oil with a density of 1010kg/m3.

The first British standard for fuel oil came in 1982. The latest standard is ISO 8217 from 2005. The ISO standard describes four qualities of distillate fuels and 10 qualities of residual fuels. Over the years the standards has become stricter on environmentally important things like sulphur content. The latest standard also banned the adding of used lubricating oil (ULO).

Some parameters of marine fuel oils according to ISO8217 (3. ed 2005):

| Marine Distillate Fuels |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Unit | Limit | DMX | DMA | DMB | DMC |
| Density at 15°C | kg/m3 | Max | - | 890.0 | 900.0 | 920.0 |
| Viscosity at 40°C | mm²/s | Max | 5.5 | 6.0 | 11.0 | 14.0 |
| Viscosity at 40°C | mm²/s | Min | 1.4 | 1.5 | - | - |
| Water | % V/V | Max | - | - | 0.3 | 0.3 |
| Sulphur1 | % (m/m) | Max | 1.0 | 1.5 | 2.0 | 2.0 |
| Aluminium + Silicon2 | mg/kg | Max | - | - | - | 25 |
| Flash point3 | °C | Min | 43 | 60 | 60 | 60 |
| Pour point, Summer | °C | Max | - | 0 | 6 | 6 |
| Pour point, Winter | °C | Max | - | -6 | 0 | 0 |
| Cloud point | °C | Max | -16 | - | - | - |
| Calculated Cetane Index |  | Min | 45 | 40 | 35 | - |

1. Max sulphur content is 1.5% in designated areas.
2. The aluminium + silicon value is used to check for remains of the catalyst after catalytic cracking. Most catalysts contain aluminium or silicon and remains of catalyst can cause damage to the engine.
3. The flash point of all fuels used in the engine room should be at least 60°C (DMX is used for things like emergency generators and not normally used in the engine room).

| Marine Residual Fuels |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Unit | Limit | RMA 30 | RMB 30 | RMD 80 | RME 180 | RMF 180 | RMG 380 | RMH 380 | RMK 380 | RMH 700 | RMK 700 |
| Density at 15°C | kg/m3 | Max | 960.0 | 975.0 | 980.0 | 991.0 | 991.0 | 991.0 | 991.0 | 1010.0 | 991.0 | 1010.0 |
| Viscosity at 50°C | mm²/s | Max | 30.0 | 30.0 | 80.0 | 180.0 | 180.0 | 380.0 | 380.0 | 380.0 | 700.0 | 700.0 |
| Water | % V/V | Max | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| Sulfur1 | % (m/m) | Max | 3.5 | 3.5 | 4.0 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 |
| Aluminium + Silicon2 | mg/kg | Max | 80 | 80 | 80 | 80 | 80 | 80 | 80 | 80 | 80 | 80 |
| Flash point3 | °C | Min | 60 | 60 | 60 | 60 | 60 | 60 | 60 | 60 | 60 | 60 |
| Pour point, Summer | °C | Max | 6 | 24 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
| Pour point, Winter | °C | Max | 0 | 24 | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 |

1. Max sulphur content is 1.5% in designated areas.
2. The aluminium + silicon value is used to check for remains of the catalyst after catalytic cracking. Most catalysts contain aluminium or silicon and remains of catalyst can cause damage to the engine.
3. The flash point of all fuels used in the engine room should be at least 60°C.