Description of Water-tube Boiler

- A water-tube boiler is a type of boiler in which water circulates in tubes which are heated externally by the fire.

- Water-tube boilers are used for high-pressure boilers.

- Fuel is burned inside the furnace, creating hot gas which heats up water in the steam-generating tubes.

- The heated water then rises into the steam drum.

- Here, saturated steam is drawn off the top of the drum.

- In a superheated boiler (the norm for propulsion boilers), the steam will re-enter the furnace in through a superheater in order to become superheated.

- Superheated steam is used in driving turbines. Since water droplets can severely damage turbine blades, steam is superheated to 730°F (390°C) or higher in order to ensure that there is no water entrained in the steam.

- Cool water at the bottom of the steam drum returns to the feedwater drum via large-bore 'downcomer tubes', where it helps pre-heat the feedwater supply.

- To increase the economy of the boiler, the exhaust gasses are also used to pre-heat the air blown into the furnace (air-preheater) and warm the feedwater supply (economizer).