Deaerator Functions

The function of a deaerator is to

- Remove dissolved oxygen,

- Remove carbon dioxide and

- Remove other non-condensable gases from boiler feedwater

- Prevent corrosion in piping, economizers, boilers and heat exchangers.

In addition, the deaerator

- Heats the boiler feedwater and

- Provides the deaerated water at saturated steam temperature to the boiler feed pumps.

Deaeration is achieved by

- Heating the water to full saturation temperature at which point the solubility of any gas is zero

- Cascading the heated water over trays to minimize the distance the liberated gas bubbles must travel to reach the surface and

- Scrubbing action of the counterflow steam, sweeping the liberated gases from the surface of the water and carrying them out the vent.