Pre-checks for Burners

- Check fuel oil strainers and the entire fuel oil system to make sure that everything is in good condition.

- Inspect the burner air casing to make sure no oil has dripped around the burners this will crate a fire hazard. If there are drip pans, then make sure.

- Check whether the burners or atomizer housing tube parts have been replaced and if the positions of the sprayer plate are correct. This setting is very important.

- Do not use fuel gas to light up a fire. After lighting up, change to gas firing. Check the entire fuel gas system according to schedule, and make sure everything is in good condition