# Avoidance and Minimization

## Introduction

State and Federal Law generally requires that the applicant demonstrate that impacts to jurisdictional wetlands and waters have been avoided and minimized to the greatest extent practicable.  Highway construction practices should be considered when evaluating alternatives.  ODOT has developed this guidance to present known approaches to avoiding and minimizing impacts to wetlands and waterways. Table 1 below summarizes, based on activity type, some common wetland impacts associated with road construction projects.

This guidance is intended for small scale projects such as bridge or culvert replacement, minor road widening (i.e., Categorical Exclusions) that typically result in minor impacts.  This document is not intended for larger projects such as roadway alignments that can result in higher permanent impacts and which may require a more in-depth analysis.  The development of mitigation plans is provided in separate guidance documents.

Note: Avoidance and minimization efforts are applicable to temporary as well as permanent impacts

## Background

State and federal regulations require that an applicant utilize a three-part process (mitigation sequencing) when evaluating potential impacts to wetlands and waterways. The mitigation sequence generally is as follows:

- Avoid adverse impacts to wetlands

- Minimize adverse impacts that cannot be avoided

- Mitigate for the remaining impacts that cannot be avoided or further minimized. Note: compensatory mitigation relates to permanent impacts and permanent loss of function, and can be used to offset significant short-term impacts that may be in effect for an extended period of time.

DSL and the Corps require documentation of avoidance and minimization measures. [Clean Water Act Section 404 (b)(1); OAR 141-085-0025(3) (j)].  In order for an alternative to be considered it should be ‘practicable,’ as defined by both DSL and the Corps as being capable of being accomplished after taking into consideration cost, existing technology, and logistics with respect to the overall project purpose.   It may be that providing compensatory mitigation is environmentally preferable and more practicable than avoiding a small wetland that is hydrologically and vegetatively degraded, thereby providing ecological uplift.

Avoidance measures should be discussed early on with the project team, including the construction office, during project development

Refer to OAR 141-085-0029(4) for more details regarding alternatives analysis requirements.

Table 1. Examples of Wetland and Water Affects

| Type of Activity | Description | Typical Examples and Effects |
| --- | --- | --- |
| Fill | Fill is defined as any organic or inorganic material that is placed in a wetland or waterway | - Temporary construction access roads - Stockpiling of construction material and or debris - Bridge demolition - Construction pads for boring or other work - Trench side cast material - Work bridge or temporary detour structure piling - Highway improvements—shoulder widening, additional lanes, guardrail flares, etc. |
| Removal | Removal is defined as any organic or inorganic material that is removed from a wetland or waterway | - Exploratory sampling or boring - Surface grading to create construction ‘pad’ or flat work area - Temporary trenching - Ditch cleaning where not exempt |
| Clearing / Grubbing | Clearing and grubbing involves the removal of vegetation and the top layer of soil to remove part of the roots. | - Sub-grade preparation - Abutment construction - Road bank re-contouring |
| Vegetation Clearing | To claim vegetation disturbance without soil disturbance the activity must involve cutting trees and shrubs off at their base, plucking them up, placing the debris in a truck and hauling it off. | - Creation of a roadway clear-zone |
| Hydrologic Changes | Changes that the project causes that affect the hydrology of the wetland or waterway [OAR 141-085-0025(3)(g)]. | - Impeding or restricting the passage of normal or expected high flows - Stormwater treatment or drainage design that effectively alters wetland hydrology - Causing flooding or erosion downstream of the project - Draining an adjacent wetland by trenching/ditching/excavating |

## Avoidance Measures

Avoiding wetlands is the first step in the mitigation sequencing. Avoidance measures should be discussed early on with the project and design team during project development. Avoidance measures should be adequately and clearly coordinated and documented during project scoping and Environmental Baseline Reports, and then later in permit applications. Be sure to include the construction office when discussing construction techniques.  Table 2 presents common activities and related avoidance measures.

Table 2. Common Avoidance Measures

| Construction and Design Elements | Considerations |
| --- | --- |
| Narrowing the Road Footprint | Exemptions or variances to road standards can be obtained for some projects. Combining sidewalks and bike lanes into a shared lane or removing planting strips or center turn lanes through wetlands and waterways can be used to minimize the road footprint and avoid impacts, if safety allows. The use walls instead of fill slopes near wetlands and waterways should be considered if feasible and sustainable (i.e., will protect wetland functions with little maintenance). Walls are usually more costly than fill but the cost of mitigation should be weighed against the cost of fill. |
| Bridge Placement | Bridges should be designed to full span and avoid waterways and high quality wetlands. If lengthening the bridge to achieve a full span is not practicable, then the number and size of piers placed in the waterway or wetland should be limited. Use of rip rap can be used where necessary but should be minimized if possible. |
| Road Alignment | Relocating the road to another location to avoid wetlands and waterways is often not feasible, however slight shifts in alignment can often avoid and or minimize impacts to wetlands and waterways. |
| Temporary Roads and Access | Consider construction staging to avoid impacts where necessary and safety allows. Place staging areas, temporary access roads, or storage areas outside of wetlands and waterways. The wetlands and waterways should be demarcated on the plans sheets as well as in the field as no work zones to avoid incidental impacts. If temporary access to piers or girders on a bridge is needed, evaluate the feasibility of conducting the work using scaffolding hanging from the topside of the bridge or limiting equipment access to the bank above OHW. |
| Stormwater Facilities | Avoid constructing storm water facilities in wetlands. Other locations or designs for stormwater facilities should be reviewed to determine that no other practicable alternative to locating them within a wetland exists. |
| Temporary Stockpiling of Materials | Avoid temporarily stockpiling construction material in wetlands. Suitable staging and stockpile areas are usually available. If absolutely necessary, follow minimization techniques for temporary fill described in Table 3. |

## Minimization Techniques

The following table (Table 3) regarding construction techniques that may minimize site disturbance should be shared with the construction office during project development. Construction techniques should be adequately described in any permit application as well as measures taken to minimize their effects.

Table 3. Minimization Measures

| Design and Construction Elements | Minimization Measure(s) |
| --- | --- |
| Temporary Fill for access roads, staging, other. | Can compact soil and also be difficult to remove after use (e.g., individual pieces of gravel sink into the soil substrate. Fill material removal can be made easier by using geotextile fabric in combination with either chain link fencing (placed below the geotextile fabric), or a geogrid (placed on top of geotextile fabric). Fill material is then placed on the top of these materials. When construction is complete fill is removed in lifts until a small amount remains to allow the sub material to be pulled up without ripping or breaking. The use of large ‘balloon’ tires on trucks and other equipment can help prevent soil compaction. |
| Construction Near Wetlands | If not properly demarcated construction impacts can “spread” into wetlands intended for avoidance. To minimize impacts clearly identify wetlands on plan sheets and demarcate on the ground using orange construction fencing and or silt fencing. Consider including a special provision under 290.41 that requires fencing to be verified by Engineer prior to mobilization in the vicinity. |
| Bridge Demolition and Piling | To minimize impacts geotextile fabric may be placed over the wetland to ensure adequate removal of smaller material. Over waterways, bridge demolition debris should be wholly contained with hard containment. |
| Construction Pads | Consider the use of crane mats, which are easily removed upon project completion with little mess (see photo). |
| Side cast material | Side cast material placed in wetlands should be replaced in the trench in the same order removed (i.e. topsoil segregation) to ensure the integrity of the soil is maintained and wetland impacts where the trench is located are minimized. |
| Piling | Piling should be removed in their entirety whenever possible. If it is not possible, or the piling breaks off during attempted removal, the piling should be cut to two feet below the wetland or waterway and the hole backfilled with native soil or streambed material. |
| Sampling or Boring | Limit access to the wetland by sampling the edges instead of driving through the middle. If access is necessary it should be timed when the area is dry. |
| Temporary Trenching | Temporary impacts can become permanent if the trench is not adequately backfilled to prevent draining the adjacent wetland. Consider using clay plugs in the trench if the trench has the potential to “dewater” a wetland. Monitor area for changes. |
| Clearing / Grubbing and Vegetation Clearing | Limits of construction should be clearly shown on plans and identified on the ground. Wetlands should not be grubbed unless necessary or where they will be permanently impacted. If appropriate re-vegetate cleared and grubbed areas. |
| Hydrologic Changes | Implement erosion control measures and manage construction storm water to avoid sedimentation in wetlands. Place permeable fill material or install small culverts to maintain connectivity between wetlands. New stormwater facilities should use infiltration or avoid redirection from or to an existing wetland or waterway. |
| Temporary Work Bridges | Use temporary work bridges that span the OHW of the waterway instead of culverts or temporary fill. If spanning isn’t an alternative, minimize piling and account for potential navigation. |
| Bridge Construction | Consider staging construction to avoid having a full detour bridge with associated approach impacts. Staging can have an adverse affect on traffic mobility and construction safety and is not always recommended. |

## Plan Sheets and Specifications

If wetlands and waterways are within the construction limits of the project then those sensitive areas must be shown on the plan sheets and demarcated in the field as no work zones using orange construction fencing or orange silt fencing, if appropriate [Special Provision 00290.41]. Wetlands and waterways that cannot be avoided (either permanent or temporary impacts) must also be shown on the plan sheets and demarcated in the field.

Special provisions should be drafted for review by the project development team and the construction office to verify their feasibility prior to sending to the specification writer for inclusion in the contract document.

The ODOT Standard Specifications and Special Provisions provide standard language for describing sensitive areas and no work zones. The Standard Specifications and Special Provisions should be reviewed to make sure that all pertinent avoidance and minimization measures are included. Bid items related to these specifications should also be included.

Unique provisions are also available for miscellaneous activities including the construction of temporary access roads through sensitive areas (00231).

Pre-Construction Meetings

Avoidance and minimization measures don’t stop when the project is handed off to construction and permits (if needed) are obtained.  It is important to review plan sheets and specifications with the Region Environmental Coordinator and the construction office (construction Project Manager and inspectors).  Before construction begins near the location of a protected resource or where special construction techniques are proposed, an on-site meeting is recommended.  The purpose of the meeting is to ensure proper placement of protective fencing, to ensure resources are adequately known and protected where necessary, and to familiarize field personnel with special provisions and applicable permit restrictions.  The number of on-site meetings will vary depending on the size of the project.

‘Impacts’ include fill, removal, mechanized land clearing (e.g., grubbing, bulldozing vegetation)

According to the U.S. Army Corps of Engineers, piling is considered a structure and the placement of piling in a wetland or waterway does not require a permit under Section 404 of the Clean Water Act, which regulates the discharge of fill material. The placement of piling may be regulated under Section 10 of the Rivers and Harbors Act, which applies to tidally influenced and navigable waterways.

Note that strictly removing piling may not be a regulated activity by the Corps, but backfilling would trigger the need for a permit, as would temporary water management techniques that involve fill (i.e., sand bags are considered fill; coffer dams are structures and are not considered as fill)